# A PROJECT REPORT ON

# ELDER FALLING DETECTION SYSTEM USING SPI PROTOCOL WITH ADXL 345 IMPLEMENTED ON FPGA BOARD

Submitted by

CH. RAM GOPAL

G. STALIN

B.C. GOVARDHAN

Under the guidelines of:

Mr.Sudheer Reddy                                                               Mr.P. Tejeswara Rao

Chairman and founder of SSIT                                            FPGA Consultant

Abstract:

Falls are among the most serious health risks for the elderly, often resulting in injury, loss of independence, or even fatality when immediate assistance is not available. With the increasing aging population, the need for reliable and real-time fall detection systems is more important than ever. According to statistics, falls are the primary cause of injury or death for the elderly over 65 years old. About 30% of the elderly over 65 years old fall every year. Along with the increase in the elderly fall accidents each year, it is urgent to find a fast and effective fall detection method to help the elderly fall.

To solve the above problem, in this paper, we propose an approach for the designing and implementation of a wearable fall detection system using the ADXL345 3-axis accelerometer interfaced with an Artix-7 FPGA (also called as Nexys A7 board) through the SPI protocol.

The system continuously monitors acceleration data to identify fall events based on a combination of free-fall detection, impact recognition, and subsequent inactivity. The detection logic is implemented in Verilog HDL using a threshold-based finite state machine (FSM) that processes real-time sensor data. Upon detecting a fall, the system triggers a visual and/or audible alert via an LED or buzzer, enabling timely response from caregivers or emergency systems.

Introduction:

As the global population continues to age, ensuring the safety and well-being of elderly individuals has become a growing concern in both healthcare and home environments. Among the various health risks faced by seniors, falls represent one of the most common and dangerous incidents. According to the World Health Organization (WHO), falls are the second leading cause of unintentional injury deaths worldwide, with individuals aged 65 and above being particularly vulnerable. About 30% of the elderly over 65 years old fall every year. In 2015, there were 29 million elderly falls in the United States, of which 37.5% required medical treatment or restricted activities for 1 day or more, and about 33,000 people died. Falls not only make the elderly suffer moderate or severe injuries, but also bring a mental burden and economic pressure to the elderly and their relatives. Faced with this situation, it is particularly important to quickly and effectively detect the fall of the elderly and provide emergency assistance. In a word, it is extremely important for those who fall and cannot call for help to be found in time and to be treated.

Traditional fall detection methods include manual alarms, wearable buttons, or camera-based monitoring systems. However, these approaches either require user intervention—which is not always feasible after a fall—or raise privacy concerns in personal spaces. To address these limitations, wearable sensor-based solutions have emerged as a more effective and privacy-preserving alternative. Among these, accelerometers play a vital role due to their ability to detect motion, impact, and orientation changes in real-time.

This project presents a real-time fall detection system using the ADXL345 accelerometer and Artix-7 FPGA (Nexys A7) via SPI communication. It uses a Verilog-coded FSM to analyse acceleration data and detect falls based on free-fall, impact, and inactivity patterns. When a fall is detected, an alert is triggered through a buzzer or LED. The system highlights how FPGA-based designs can provide fast, reliable, and privacy-friendly solutions for wearable health monitoring.

Objectives:

- Establish Communication Between ADXL345 and FPGA via SPI Protocol
The first objective is to set up reliable and continuous communication between the ADXL345 accelerometer and the Artix-7 FPGA using the SPI protocol. This includes writing Verilog code for an SPI master that can read and write to the sensor’s internal registers.

- Configure the ADXL345 for Real-Time Acceleration Measurement
Proper configuration of the ADXL345 is critical. This includes writing to its control registers to enable measurement mode, setting the output data format, and defining the measurement range (e.g., ±2g or ±4g), so the sensor can accurately capture sudden motion changes associated with a fall.

- Acquire and Process 16-bit Signed Acceleration Data from X, Y, and Z Axes
The FPGA should continuously fetch the raw acceleration values from the sensor, correctly combine the low and high bytes for each axis, and perform sign extension to preserve the signed 16-bit format. This processed data will serve as the core input for fall detection.

- Implement a Threshold-Based Fall Detection Algorithm in Verilog
Using the acceleration data, a logic module (FSM) will analyse the magnitude and patterns to detect fall events. The detection logic should identify sequences like free-fall, sudden impact, and subsequent inactivity, which are typical indicators of a fall.

- Trigger Visual or Auditory Alerts When a Fall is Detected
Upon detecting a fall, the system must immediately activate a buzzer or light up an LED to alert caregivers or nearby individuals. This objective ensures the system can respond instantly and visibly in real-world situations.

- Achieve Real-Time and Low-Latency Operation Using FPGA Hardware
By implementing the entire detection mechanism in hardware (Verilog on FPGA), the system should be able to process inputs and make decisions within milliseconds, unlike software systems that may introduce lag. This objective is vital for time-sensitive medical response.

Hardware Requirements:

Artix-7 FPGA Development Board :

The Artix-7 FPGA family from Xilinx is manufactured using a cutting-edge 28nm high-performance, low-power (HPL) CMOS process, which provides an excellent balance of power efficiency and high logic density. This makes the Artix-7 ideal for applications that require low power consumption without sacrificing computational power. Depending on the specific device model, the Artix-7 offers a wide range of resources, including up to 215,360 logic cells, 740 DSP slices for intensive digital signal processing, and as much as 13,800 Kbits of fast block RAM. These features allow it to handle both simple control tasks and complex, high-speed computations effectively.

In addition to its computational resources, the Artix-7 supports a versatile set of memory and interface options. It is compatible with DDR3 and DDR3L memory standards, enabling high-bandwidth data storage and retrieval. The FPGA also features numerous user-accessible I/O pins, often exceeding 500, which support multiple communication protocols such as SPI, UART, I²C, and high-speed LVDS serial interfaces. These interfaces ensure seamless connectivity with external sensors, memory modules, and communication devices.

One of the notable built-in peripherals is the XADC (Xilinx Analog-to-Digital Converter), a 12-bit ADC capable of sampling analog signals at up to 1 million samples per second. This enables the Artix-7 to perform mixed-signal processing and monitor analog inputs without requiring external ADC components

A popular implementation of the Artix-7 FPGA is found on the Digilent Nexys A7 development board, which uses the XC7A100T-1CSG324C chip. This particular FPGA model provides 15,850 logic slices, 240 DSP slices, and 4,860 Kbits of block RAM, along with access to over 200 user I/O pins. Additional onboard components like LEDs, pushbuttons, switches, 7-segment displays, VGA output, and audio codec support make it an excellent platform for prototyping diverse embedded systems.Overall, the Artix-7 FPGA series stands out as a cost-effective, power-efficient, and highly flexible platform for modern digital design. Its combination of high logic capacity, rich peripheral support, and efficient power management makes it ideal for applications such as real-time signal processing, embedded control systems, and advanced sensor interfacing, including projects like fall detection systems.

ADXL345 Accelerometer Module:

The ADXL345 is a compact, low-power, 3-axis digital accelerometer manufactured by Analog Devices. Built with advanced microelectromechanical system (MEMS) technology, it provides high-resolution motion sensing in a small footprint, making it an excellent choice for embedded systems that require precise measurement of acceleration, tilt, and vibration.

This sensor can measure acceleration with a selectable range of ±2g, ±4g, ±8g, and ±16g, allowing for flexibility in detecting both subtle movements and rapid motions. It outputs data digitally via either a standard I²C or SPI interface, facilitating easy integration with microcontrollers, FPGAs, or other digital platforms. The ADXL345 supports a data rate from 0.1 Hz up to 3200 Hz, providing fast and responsive motion tracking suitable for real-time applications.

One of the ADXL345’s strengths lies in its onboard 13-bit analog-to-digital converter (ADC), which ensures accurate and high-resolution readings. Combined with its programmable interrupts and configurable thresholds, it can detect events like free-fall, tap, double tap, and activity/inactivity, offloading these processing tasks from the host processor and improving system efficiency.

The sensor’s low power consumption (typically around 40 µA during measurement mode) makes it highly suitable for battery-powered and portable devices. Its supply voltage ranges from 2.0 to 3.6 V, allowing compatibility with common embedded power rails.

Commonly used in applications such as motion-based gesture recognition, device orientation sensing, and fall detection systems, the ADXL345’s compact size, versatile interface options, and advanced features make it a popular choice for developers aiming to add motion awareness to their projects.

The ADXL345 accelerometer combines high accuracy, flexible measurement ranges, and power-efficient operation within a small MEMS package. These characteristics make it ideal for applications ranging from wearable devices and gaming controllers to healthcare monitoring systems, where precise and reliable motion sensing is critical.

Buzzer or LED for Alert Output

Used to give a clear visual or audible signal when a fall is detected by the system.

- Jumper Wires
Required to connect the sensor and output devices to the appropriate pins on the FPGA board.

- Breadboard
Helps in testing and organizing connections without soldering during the development and debugging phase.

Software Required:

- Xilinx Vivado Design Suite
Used for writing Verilog code, synthesizing the design, assigning pin constraints, and programming the FPGA.

- Intel Quartus Prime (if you are using DE-10 NANO board)
This is the primary development environment used to write, compile, and synthesize Verilog code, assign pin constraints, and program the DE10-Nano FPGA.

- Verilog HDL
The hardware description language used to develop the SPI communication module, ADXL345 interface logic, fall detection FSM, and alert control logic

.

Working principle:

The Elder Fall Detection System uses a digital 3-axis accelerometer (ADXL345) interfaced with the FPGA (here the FPGA is Artix-7) to detect sudden or sustained orientation changes that indicate a fall. The system continuously monitors the Z-axis acceleration values and activates an alert if the Z-axis value remains below a set threshold for a specified time duration (e.g., 3 seconds), suggesting the person is lying down after a fall.

### 1. Sensor Initialization via SPI

The FPGA initializes the ADXL345 sensor using SPI communication:

- The SPI master module sends a write command to enable measurement mode (0x02 to register 0x2D).

- It then enters a continuous loop of burst reads starting at register 0x0E to collect X, Y, and Z axis data.

### 2. Data Collection

- The SPI master captures 16-bit signed data for each axis.

- For simplicity, the data is reduced to 5 significant bits per axis (configurable) and latched into an output register accelerometer data at every read cycle (~100 Hz).

### 3. Fall Detection Logic

- The top module reads accelerometer data and sign-extends the Z-axis component to a full 16-bit signed value.

- It checks if z axis data is continuously less than a threshold (e.g., -900), which indicates the person is likely on the ground (low vertical acceleration).

- A counter increments for every clock cycle (assumed 100 MHz) while the Z value remains below the threshold.

- If the condition persists for 3 seconds (i.e., 300 million clock cycles), the system activates a buzzer to signal a possible fall incident.

### 4. Reset Behaviour

- If the Z value rises above the threshold (indicating movement or recovery), the counter and buzzer are reset immediately, ensuring no false alarms if the person is just briefly idle.

System Design:

a. Block diagram:

ADXL345 Accelerometer

|

SPI

|

FPGA (Artix-7)

┌───────────────┐

│ Fall Detection│

│   FSM Logic   │

└───────────────┘

|

ALERT SYSTEM

(LED / Buzzer)

b. Sensor Interface:

- Communication Protocol: SPI

- SPI mode: 3

- Device Address: SPI-based chip select (no I2C address)

- Important Registers:

  - POWER_CTL (0x2D): Set to 0x08 to enable measurement mode

  - DATA_FORMAT (0x31): Configured for ±2g or ±4g range

  - DATAX0 – DATAZ1 (0x32 – 0x37): 16-bit signed axis data

c. FSM flow:

The FSM starts at the power-up state (S0) with a 6 ms wait for sensor stabilization. In S1, CS is pulled low to begin SPI communication. The FSM then sends a write command (0x0A), the POWER_CTL register address (0x2D), and the data byte 0x02 to enable measurement mode (states S2–S25). After setting CS high in S26 and waiting 40 ms, the FSM moves to the read phase. It pulls CS low again (S27), sends a read command (0x0B) and the data start address (0x0E) (states S28–S43). The FSM then reads 16-bit acceleration data for X (S44–S59), Y (S60–S75), and Z (S76–S91) axes. After reading, it sets CS high (S92), waits 10 ms, and loops back to the read phase for continuous data acquisition.

OPERATION OF ELDER FALLING SYSTEM:

State Machine Flow for SPI Communications with 3-Axis Accelerometer

1. Wait for 6ms to allow sensor to power up and go into standby mode

2. Send a write instruction (0x0A)

3. Send the register address to write to (0x2D)

4. Send the data to be written (0x02) <-- sets mode to measurement mode

5. Wait for 40ms to allow sensor to calculate valid data after being put into measurement mode

6. Send a read instruction (0x0B)

7. Send the register address to read from (0x0E) <-- LSB of X axis data

8. Read X axis LSB

9. Read X axis MSB

10. Read Y axis LSB

11. Read Y axis MSB

12. Read Z axis LSB

13. Read Z axis MSB

14. Wait for 10ms to allow calculation of another set of valid data, loop back to 6.

Notes about the sensor retrieved from the ADXL362 datasheet:

- The sensor defaults to standby mode 5ms after power up

- No measurement functions are active in standby mode

- After putting sensor in measurement mode it takes 40ms to produce first data set

- The default data rate of the sensor is 100Hz <-- 10ms for a new data set

- The SPI Mode of the sensor is CPOL = CPHA = 0 <-- SPI Mode 0

- The max. SCLK for the sensor is 8MHz, min. is 12.5kHz

- Recommended SCLK speed is 1MHz to 8MHz

- For a burst read only the address of the first register to read from is needed, the

register address pointer in the sensor is auto-incremented to the next register in

hex order couting up until communications have been disabled by the master

- The sensor only reacts to 3 different instructions:

- 0x0A : write register

- 0x0B : read register

- 0x0D : read FIFO (for more information on the FIFO, see the datasheet)

IMPLEMENT SYNTHESIS

CODE

module spi_master(

input iclk,                 // 4MHz

input miso,                 // master in

output sclk,				// 1MHz

output reg mosi = 1'b0,     // master out

output reg cs = 1'b1,       // slave chip select

output [14:0] acl_data     // 15 bit data, 5 each axis

);

// Control sclk output for spi mode

reg sclk_control = 1'b0;

// Create a 1MHz signal from a 4MHz signal

// For a 50% duty cycle

// 4 x 10^6 / 1 x 10^6 / 2 = 2

reg clk_counter = 1'b0;

reg clk_reg = 1'b1;

always @(posedge iclk) begin

clk_counter <= clk_counter + 1;

if(clk_counter == 1'b1)

clk_reg <= ~clk_reg;         // this is sclk

end

reg [7:0] write_instr   = 8'h0A;			// Sensor write instruction

reg [7:0] mode_reg_addr = 8'h2D;			// Mode register address

reg [7:0] mode_wr_data  = 8'h02;			// Data to write to mode register

reg [7:0] read_instr    = 8'h0B;			// Sensor read instruction

reg [7:0] x_LSB_addr    = 8'h0E;			// X data LSB address, auto-increments to following 5 data registers

reg [14:0] temp_DATA    = 15'b0;			// 15 bits, 5 bits for each axis

reg [15:0] X = 15'b0;						// X data MSB and LSB from sensor

reg [15:0] Y = 15'b0;					    // Y data MSB and LSB from sensor

reg [15:0] Z = 15'b0;						// Z data MSB and LSB from sensor

reg [31:0] counter = 32'b0;         		// State machine sync counter - 4000 ticks per ms at 4MHz

wire latch_data;

// 93 states for state machine

localparam [6:0] POWER_UP    = 7'h00,		// Wait 6ms, CS = 0, SCLK = idle = 0

// SPI WRITE

BEGIN_SPIW  = 7'h01,

// Send write instruction 0x0A

SEND_WCMD7   = 7'h02,

SEND_WCMD6   = 7'h03,

SEND_WCMD5   = 7'h04,

SEND_WCMD4   = 7'h05,

SEND_WCMD3   = 7'h06,

SEND_WCMD2   = 7'h07,

SEND_WCMD1   = 7'h08,

SEND_WCMD0   = 7'h09,

// Send register address to write to 0x2D

SEND_WADDR7  = 7'h0A,

SEND_WADDR6  = 7'h0B,

SEND_WADDR5  = 7'h0C,

SEND_WADDR4  = 7'h0D,

SEND_WADDR3  = 7'h0E,

SEND_WADDR2  = 7'h0F,

SEND_WADDR1  = 7'h10,

SEND_WADDR0  = 7'h11,

// Send byte to put into measurement mode 0x02

SEND_BYTE7  = 7'h12,

SEND_BYTE6  = 7'h13,

SEND_BYTE5  = 7'h14,

SEND_BYTE4  = 7'h15,

SEND_BYTE3  = 7'h16,

SEND_BYTE2  = 7'h17,

SEND_BYTE1  = 7'h18,

SEND_BYTE0  = 7'h19,

// Wait for first valid data after init measurement mode = 40ms

WAIT        = 7'h1A,

// SPI READ

BEGIN_SPIR  = 7'h1B,

// Send read instruction 0x0B

SEND_RCMD7  = 7'h1C,

SEND_RCMD6  = 7'h1D,

SEND_RCMD5  = 7'h1E,

SEND_RCMD4  = 7'h1F,

SEND_RCMD3  = 7'h20,

SEND_RCMD2  = 7'h21,

SEND_RCMD1  = 7'h22,

SEND_RCMD0  = 7'h23,

// Send X data LSB register address 0x0E

SEND_RADDR7 = 7'h24,

SEND_RADDR6 = 7'h25,

SEND_RADDR5 = 7'h26,

SEND_RADDR4 = 7'h27,

SEND_RADDR3 = 7'h28,

SEND_RADDR2 = 7'h29,

SEND_RADDR1 = 7'h2A,

SEND_RADDR0 = 7'h2B,

// Receive X data LSB from 0x0E

REC_XLSB7	 = 7'h2C,

REC_XLSB6	 = 7'h2D,

REC_XLSB5	 = 7'h2E,

REC_XLSB4	 = 7'h2F,

REC_XLSB3	 = 7'h30,

REC_XLSB2	 = 7'h31,

REC_XLSB1	 = 7'h32,

REC_XLSB0	 = 7'h33,

// Receive X data MSB from 0x0F

REC_XMSB7	 = 7'h34,

REC_XMSB6	 = 7'h35,

REC_XMSB5	 = 7'h36,

REC_XMSB4	 = 7'h37,

REC_XMSB3	 = 7'h38,

REC_XMSB2	 = 7'h39,

REC_XMSB1	 = 7'h3A,

REC_XMSB0	 = 7'h3B,

// Receive Y data LSB from 0x10

REC_YLSB7	 = 7'h3C,

REC_YLSB6	 = 7'h3D,

REC_YLSB5	 = 7'h3E,

REC_YLSB4	 = 7'h3F,

REC_YLSB3	 = 7'h40,

REC_YLSB2	 = 7'h41,

REC_YLSB1	 = 7'h42,

REC_YLSB0	 = 7'h43,

// Receive Y data MSB from 0x11

REC_YMSB7	 = 7'h44,

REC_YMSB6	 = 7'h45,

REC_YMSB5	 = 7'h46,

REC_YMSB4	 = 7'h47,

REC_YMSB3	 = 7'h48,

REC_YMSB2	 = 7'h49,

REC_YMSB1	 = 7'h4A,

REC_YMSB0	 = 7'h4B,

// Receive Z data LSB from 0x12

REC_ZLSB7	 = 7'h4C,

REC_ZLSB6	 = 7'h4D,

REC_ZLSB5	 = 7'h4E,

REC_ZLSB4	 = 7'h4F,

REC_ZLSB3	 = 7'h50,

REC_ZLSB2	 = 7'h51,

REC_ZLSB1	 = 7'h52,

REC_ZLSB0	 = 7'h53,

// Receive Z data MSB from 0x13

REC_ZMSB7	 = 7'h54,

REC_ZMSB6	 = 7'h55,

REC_ZMSB5	 = 7'h56,

REC_ZMSB4	 = 7'h57,

REC_ZMSB3	 = 7'h58,

REC_ZMSB2	 = 7'h59,

REC_ZMSB1	 = 7'h5A,

REC_ZMSB0	 = 7'h5B,

// End SPI communications

END_SPI	 = 7'h5C;	// Wait 10ms, loop back to BEGIN_SPIR

// State Register

reg [6:0] state_reg = POWER_UP; // Initial state of state register

always @(posedge iclk) begin

counter <= counter + 1; // Increment state machine sync counter

case(state_reg)

POWER_UP    : begin

if(counter == 32'd23999)	// Wait for 6ms, for sensor to reach standby mode

state_reg <= BEGIN_SPIW;

end

// Begin SPI write communication with sensor by sending CS line low

BEGIN_SPIW	: begin						// 24000

if(counter == 32'd24001) begin

state_reg <= SEND_WCMD7;

cs <= 1'b0;			// 24002

end

end

// Send the write command to initiate a write sequence

SEND_WCMD7    : begin					// 24002

sclk_control <= 1'b1;	// 24003

mosi <= write_instr[7];	// 24003

if(counter == 32'd24005)

state_reg <= SEND_WCMD6;

end

SEND_WCMD6    : begin					// 24006

mosi <= write_instr[6];	// 24007

if(counter == 32'd24009)

state_reg <= SEND_WCMD5;

end

SEND_WCMD5    : begin					// 24010

mosi <= write_instr[5];	// 24011

if(counter == 32'd24013)

state_reg <= SEND_WCMD4;

end

SEND_WCMD4    : begin					// 24014

mosi <= write_instr[4];	// 24015

if(counter == 32'd24017)

state_reg <= SEND_WCMD3;

end

SEND_WCMD3    : begin					// 24018

mosi <= write_instr[3];	// 24019

if(counter == 32'd24021)

state_reg <= SEND_WCMD2;

end

SEND_WCMD2    : begin					// 24022

mosi <= write_instr[2];	// 24023

if(counter == 32'd24025)

state_reg <= SEND_WCMD1;

end

SEND_WCMD1    : begin					// 24026

mosi <= write_instr[1];	// 24027

if(counter == 32'd24029)

state_reg <= SEND_WCMD0;

end

SEND_WCMD0    : begin					// 24030

mosi <= write_instr[0];	// 24031

if(counter == 32'd24033)

state_reg <= SEND_WADDR7;

end

// Send the address to write to, in this case the 0x2D register, to configure sensor to measurement mode

SEND_WADDR7   : begin						// 24034

mosi <= mode_reg_addr[7];	// 24035

if(counter == 32'd24037)

state_reg <= SEND_WADDR6;

end

SEND_WADDR6   : begin						// 24038

mosi <= mode_reg_addr[6];	// 24039

if(counter == 32'd24041)

state_reg <= SEND_WADDR5;

end

SEND_WADDR5   : begin						// 24042

mosi <= mode_reg_addr[5];	// 24043

if(counter == 32'd24045)

state_reg <= SEND_WADDR4;

end

SEND_WADDR4   : begin						// 24046

mosi <= mode_reg_addr[4];	// 24047

if(counter == 32'd24049)

state_reg <= SEND_WADDR3;

end

SEND_WADDR3   : begin						// 24050

mosi <= mode_reg_addr[3];	// 24051

if(counter == 32'd24053)

state_reg <= SEND_WADDR2;

end

SEND_WADDR2   : begin						// 24054

mosi <= mode_reg_addr[2];	// 24055

if(counter == 32'd24057)

state_reg <= SEND_WADDR1;

end

SEND_WADDR1   : begin						// 24058

mosi <= mode_reg_addr[1];	// 24059

if(counter == 32'd24061)

state_reg <= SEND_WADDR0;

end

SEND_WADDR0   : begin						// 24062

mosi <= mode_reg_addr[0];	// 24063

if(counter == 32'd24065)

state_reg <= SEND_BYTE7;

end

// Send the value to write to the 0x2D register, in this case 0x02, for measurement mode

SEND_BYTE7	: begin							// 24066

mosi <= mode_wr_data[7];	// 24067

if(counter == 32'd24069)

state_reg <= SEND_BYTE6;

end

SEND_BYTE6	: begin							// 24070

mosi <= mode_wr_data[6];	// 24071

if(counter == 32'd24073)

state_reg <= SEND_BYTE5;

end

SEND_BYTE5	: begin							// 24074

mosi <= mode_wr_data[5];	// 24075

if(counter == 32'd24077)

state_reg <= SEND_BYTE4;

end

SEND_BYTE4	: begin							// 24078

mosi <= mode_wr_data[4];	// 24079

if(counter == 32'd24081)

state_reg <= SEND_BYTE3;

end

SEND_BYTE3	: begin							// 24082

mosi <= mode_wr_data[3];	// 24083

if(counter == 32'd24085)

state_reg <= SEND_BYTE2;

end

SEND_BYTE2	: begin							// 24086

mosi <= mode_wr_data[2];	// 24087

if(counter == 32'd24089)

state_reg <= SEND_BYTE1;

end

SEND_BYTE1	: begin							// 24090

mosi <= mode_wr_data[1];	// 24091

if(counter == 32'd24093)

state_reg <= SEND_BYTE0;

end

SEND_BYTE0	: begin							// 24094

mosi <= mode_wr_data[0];	// 24095

if(counter == 32'd24097) begin

state_reg <= WAIT;

counter <= 32'd0;

cs <= 1'b1;			// Turn OFF CS

sclk_control <= 1'b0;	// Turn OFF SCLK

end

end

// Wait for 40ms after setting measurement mode to allow first valid data, 160,000 ticks + 3 to line up SCLK

WAIT		: begin							// 0

if(counter == 32'd160002) begin

counter <= 32'd0;

state_reg <= BEGIN_SPIR;

end

end

// Begin SPI read communication with sensor by sending CS low

BEGIN_SPIR	: begin							// 0

if(counter == 32'd1) begin

state_reg <= SEND_RCMD7;

cs <= 1'b0;				// 2

sclk_control <= 1'b1;	// 2

end

end

// Send the read command to initiate a read sequence

SEND_RCMD7 	: begin							// 2

mosi <= read_instr[7];		// 3

if(counter == 32'd4)

state_reg <= SEND_RCMD6;

end

SEND_RCMD6 	: begin							// 5

mosi <= read_instr[6];		// 6

if(counter == 32'd8)

state_reg <= SEND_RCMD5;

end

SEND_RCMD5 	: begin							// 9

mosi <= read_instr[5];		// 10

if(counter == 32'd12)

state_reg <= SEND_RCMD4;

end

SEND_RCMD4 	: begin							// 13

mosi <= read_instr[4];		// 14

if(counter == 32'd16)

state_reg <= SEND_RCMD3;

end

SEND_RCMD3 	: begin							// 17

mosi <= read_instr[3];		// 18

if(counter == 32'd20)

state_reg <= SEND_RCMD2;

end

SEND_RCMD2 	: begin							// 21

mosi <= read_instr[2];		// 22

if(counter == 32'd24)

state_reg <= SEND_RCMD1;

end

SEND_RCMD1 	: begin							// 25

mosi <= read_instr[1];		// 26

if(counter == 32'd28)

state_reg <= SEND_RCMD0;

end

SEND_RCMD0 	: begin							// 29

mosi <= read_instr[0];		// 30

if(counter == 32'd32)

state_reg <= SEND_RADDR7;

end

// Send register address to read from, in this case 0x0E, the x data LSB

SEND_RADDR7 : begin							// 33

mosi <= x_LSB_addr[7];		// 34

if(counter == 32'd36)

state_reg <= SEND_RADDR6;

end

SEND_RADDR6 : begin							// 37

mosi <= x_LSB_addr[6];		// 38

if(counter == 32'd40)

state_reg <= SEND_RADDR5;

end

SEND_RADDR5 : begin							// 41

mosi <= x_LSB_addr[5];		// 42

if(counter == 32'd44)

state_reg <= SEND_RADDR4;

end

SEND_RADDR4 : begin							// 45

mosi <= x_LSB_addr[4];		// 46

if(counter == 32'd48)

state_reg <= SEND_RADDR3;

end

SEND_RADDR3 : begin							// 49

mosi <= x_LSB_addr[3];		// 50

if(counter == 32'd52)

state_reg <= SEND_RADDR2;

end

SEND_RADDR2 : begin							// 53

mosi <= x_LSB_addr[2];		// 54

if(counter == 32'd56)

state_reg <= SEND_RADDR1;

end

SEND_RADDR1 : begin							// 57

mosi <= x_LSB_addr[1];		// 58

if(counter == 32'd60)

state_reg <= SEND_RADDR0;

end

SEND_RADDR0 : begin							// 61

mosi <= x_LSB_addr[0];		// 62

if(counter == 32'd64)

state_reg <= REC_XLSB7;

end

// Receive x data LSB[7:0], store in LSB of X data reg

REC_XLSB7 	: begin							// 65

X[7] <= miso;				// 66

if(counter == 32'd68)

state_reg <= REC_XLSB6;

end

REC_XLSB6 	: begin							// 69

X[6] <= miso;				// 70

if(counter == 32'd72)

state_reg <= REC_XLSB5;

end

REC_XLSB5 	: begin							// 73

X[5] <= miso;				// 74

if(counter == 32'd76)

state_reg <= REC_XLSB4;

end

REC_XLSB4 	: begin							// 77

X[4] <= miso;				// 78

if(counter == 32'd80)

state_reg <= REC_XLSB3;

end

REC_XLSB3 	: begin							// 81

X[3] <= miso;				// 82

if(counter == 32'd84)

state_reg <= REC_XLSB2;

end

REC_XLSB2 	: begin							// 85

X[2] <= miso;				// 86

if(counter == 32'd88)

state_reg <= REC_XLSB1;

end

REC_XLSB1 	: begin							// 89

X[1] <= miso;				// 90

if(counter == 32'd92)

state_reg <= REC_XLSB0;

end

REC_XLSB0 	: begin							// 93

X[0] <= miso;				// 94

if(counter == 32'd96)

state_reg <= REC_XMSB7;

end

// Receive x data MSB[7:0], store in MSB of X data reg

REC_XMSB7 	: begin							// 97

X[15] <= miso;				// 98

if(counter == 32'd100)

state_reg <= REC_XMSB6;

end

REC_XMSB6 	: begin							// 101

X[14] <= miso;				// 102

if(counter == 32'd104)

state_reg <= REC_XMSB5;

end

REC_XMSB5 	: begin							// 105

X[13] <= miso;				// 106

if(counter == 32'd108)

state_reg <= REC_XMSB4;

end

REC_XMSB4 	: begin							// 109

X[12] <= miso;				// 110

if(counter == 32'd112)

state_reg <= REC_XMSB3;

end

REC_XMSB3 	: begin							// 113

X[11] <= miso;				// 114

if(counter == 32'd116)

state_reg <= REC_XMSB2;

end

REC_XMSB2 	: begin							// 117

X[10] <= miso;				// 118

if(counter == 32'd120)

state_reg <= REC_XMSB1;

end

REC_XMSB1 	: begin							// 121

X[9] <= miso;				// 122

if(counter == 32'd124)

state_reg <= REC_XMSB0;

end

REC_XMSB0 	: begin							// 125

X[8] <= miso;				// 126

if(counter == 32'd128)

state_reg <= REC_YLSB7;

end

// Receive y data LSB[7:0], store in LSB of Y data reg

REC_YLSB7 	: begin							// 129

Y[7] <= miso;				// 130

if(counter == 32'd132)

state_reg <= REC_YLSB6;

end

REC_YLSB6 	: begin							// 133

Y[6] <= miso;				// 134

if(counter == 32'd136)

state_reg <= REC_YLSB5;

end

REC_YLSB5 	: begin							// 137

Y[5] <= miso;				// 138

if(counter == 32'd140)

state_reg <= REC_YLSB4;

end

REC_YLSB4 	: begin							// 141

Y[4] <= miso;				// 142

if(counter == 32'd144)

state_reg <= REC_YLSB3;

end

REC_YLSB3 	: begin							// 145

Y[3] <= miso;				// 146

if(counter == 32'd148)

state_reg <= REC_YLSB2;

end

REC_YLSB2 	: begin							// 149

Y[2] <= miso;				// 150

if(counter == 32'd152)

state_reg <= REC_YLSB1;

end

REC_YLSB1 	: begin							// 153

Y[1] <= miso;				// 154

if(counter == 32'd156)

state_reg <= REC_YLSB0;

end

REC_YLSB0 	: begin							// 157

Y[0] <= miso;				// 158

if(counter == 32'd160)

state_reg <= REC_YMSB7;

end

// Receive y data MSB[7:0], store in MSB of Y data reg

REC_YMSB7 	: begin							// 161

Y[15] <= miso;				// 162

if(counter == 32'd164)

state_reg <= REC_YMSB6;

end

REC_YMSB6 	: begin							// 165

Y[14] <= miso;				// 166

if(counter == 32'd168)

state_reg <= REC_YMSB5;

end

REC_YMSB5 	: begin							// 169

Y[13] <= miso;				// 170

if(counter == 32'd172)

state_reg <= REC_YMSB4;

end

REC_YMSB4 	: begin							// 173

Y[12] <= miso;				// 174

if(counter == 32'd176)

state_reg <= REC_YMSB3;

end

REC_YMSB3 	: begin							// 177

Y[11] <= miso;				// 178

if(counter == 32'd180)

state_reg <= REC_YMSB2;

end

REC_YMSB2 	: begin							// 181

Y[10] <= miso;				// 182

if(counter == 32'd184)

state_reg <= REC_YMSB1;

end

REC_YMSB1 	: begin							// 185

Y[9] <= miso;				// 186

if(counter == 32'd188)

state_reg <= REC_YMSB0;

end

REC_YMSB0 	: begin							// 189

Y[8] <= miso;				// 190

if(counter == 32'd192)

state_reg <= REC_ZLSB7;

end

// Receive z data LSB[7:0], store in LSB of Z data reg

REC_ZLSB7 	: begin							// 193

Z[7] <= miso;				// 194

if(counter == 32'd196)

state_reg <= REC_ZLSB6;

end

REC_ZLSB6 	: begin							// 197

Z[6] <= miso;				// 198

if(counter == 32'd200)

state_reg <= REC_ZLSB5;

end

REC_ZLSB5 	: begin							// 201

Z[5] <= miso;				// 202

if(counter == 32'd204)

state_reg <= REC_ZLSB4;

end

REC_ZLSB4 	: begin							// 205

Z[4] <= miso;				// 206

if(counter == 32'd208)

state_reg <= REC_ZLSB3;

end

REC_ZLSB3 	: begin							// 209

Z[3] <= miso;				// 210

if(counter == 32'd212)

state_reg <= REC_ZLSB2;

end

REC_ZLSB2 	: begin							// 213

Z[2] <= miso;				// 214

if(counter == 32'd216)

state_reg <= REC_ZLSB1;

end

REC_ZLSB1 	: begin							// 217

Z[1] <= miso;				// 218

if(counter == 32'd220)

state_reg <= REC_ZLSB0;

end

REC_ZLSB0 	: begin							// 221

Z[0] <= miso;				// 222

if(counter == 32'd224)

state_reg <= REC_ZMSB7;

end

// Receive z data MSB[7:0], store in MSB of Z data reg

REC_ZMSB7 	: begin							// 225

Z[15] <= miso;				// 226

if(counter == 32'd228)

state_reg <= REC_ZMSB6;

end

REC_ZMSB6 	: begin							// 229

Z[14] <= miso;				// 230

if(counter == 32'd232)

state_reg <= REC_ZMSB5;

end

REC_ZMSB5 	: begin							// 233

Z[13] <= miso;				// 234

if(counter == 32'd236)

state_reg <= REC_ZMSB4;

end

REC_ZMSB4 	: begin							// 237

Z[12] <= miso;				// 238

if(counter == 32'd240)

state_reg <= REC_ZMSB3;

end

REC_ZMSB3 	: begin							// 241

Z[11] <= miso;				// 242

if(counter == 32'd244)

state_reg <= REC_ZMSB2;

end

REC_ZMSB2 	: begin							// 245

Z[10] <= miso;				// 246

if(counter == 32'd248)

state_reg <= REC_ZMSB1;

end

REC_ZMSB1 	: begin							// 249

Z[9] <= miso;				// 250

if(counter == 32'd252)

state_reg <= REC_ZMSB0;

end

REC_ZMSB0 	: begin							// 253

Z[8] <= miso;				// 254

if(counter == 32'd256) begin

cs <= 1'b1;	// disable chip select

sclk_control <= 1'b0;	// disable sclk

state_reg <= END_SPI;

end

end

// End read communications, wait 10ms for 100Hz data rate = 40,000 ticks

END_SPI 	: begin							// 257

if(counter == 32'd40259) begin

counter <= 32'd0;	// reset counter

state_reg <= BEGIN_SPIR;

end

end

endcase

end

// Data Buffer

always @(negedge iclk)

if(latch_data) begin		// latch 1&1/2 tick after entering state END_SPI

temp_DATA <= { X[11:7], Y[11:7], Z[11:7] };   // latch sign bit + 4 data bits each axis

end

// Output accelerometer data

assign acl_data = temp_DATA;

assign latch_data = ((state_reg == END_SPI) && (counter == 32'd258)) ? 1 : 0;

assign sclk = (sclk_control) ? clk_reg : 0;

endmodule

module top_module (

input clk,              // 100 MHz system clock

input rst,              // Reset signal

input miso,             // SPI MISO from ADXL345

output sclk,            // SPI clock

output mosi,            // SPI MOSI

output cs,              // SPI Chip Select

output reg buzzer,      // Buzzer output

output [14:0] acl_data

);

// Threshold for detecting fall (tuned as needed)

parameter THRESH_FREEFALL = 16'd10;

parameter THREE_SECONDS = 300_000_000;  // 3 seconds @ 100 MHz

wire [4:0] z_data_short = acl_data[4:0];  // 5-bit Z data from spi_master

wire signed [15:0] z_data_ext = { {11{z_data_short[4]}}, z_data_short }; // sign-extend to 16-bit

reg [31:0] fall_counter = 0;

// Instantiate SPI Master

spi_master accel_reader (

.iclk(clk),        // Assuming clk is 4MHz (adjust if not)

.miso(miso),

.sclk(sclk),

.mosi(mosi),

.cs(cs),

.acl_data(acl_data)

);

// Fall detection logic

always @(posedge clk or posedge rst) begin

if (rst) begin

fall_counter <= 0;

buzzer <= 0;

end else begin

if (z_data_ext < THRESH_FREEFALL) begin

if (fall_counter < THREE_SECONDS)

fall_counter <= fall_counter + 1;

else

buzzer <= 1;

end else begin

fall_counter <= 0;

buzzer <= 0;

end

end

end

endmodule

RTL For Elder Falling Detection:

RTL OF ELDER FALLING DETECTION

CONSTRAINS

RESULT:

HARDWARE COMMUNICATION:

CONCLUSION:

This project shows how we can build a real-time fall detection system using the ADXL345 sensor and an FPGA board like the Artix-7 or DE10-Nano. The system constantly checks for sudden movements and falls by reading data from the sensor and processing it using logic written in Verilog. When it detects a fall, it immediately turns on a buzzer or LED to alert someone nearby. The setup is fast, reliable, and doesn’t invade privacy, making it great for use in wearable devices or smart homes for elderly care. It also has room to grow—you can add features like wireless alerts or smarter detection using AI in the future.